/*!

\page close_contour_operation_page Close Contour

\n To produce a <b>Close Contour</b> operation in the <b>Main Menu</b>

select <b>Repair - > Close Contour</b>.

\n This operation closes an open contour and modifies the underlying

face (if needed) in accordance with user specified mode:

<ul>

<li>By common vertex &ndash; a vertex is created between the end points

of the contour and its tolerance is increased to a value of the gap

between the ends of the contour;</li>

<li>By new edge &ndash; a new edge is inserted between the end points of the contour.</li>

</ul>

This operation is available in <b>OCC Viewer</b> only.

\n The \b Result will be a \b GEOM_Object.

\n <b>TUI Command:</b> <em>geompy.CloseContour(Shape, Wires,

IsCommonVertex),</em> where \em Shape is a shape to be processed, \em Wires is

a list of edges or wires ID's which has to be closed within the

shape (if the list contains only one element = -1, the shape itself is

considered as a wire), \em IsCommonVertex if this parameter is True a

closure has to be done by creation of a common vertex, otherwise an

edge is added between the end vertices.

\n <b>Arguments:</b> Name + 1 shape + contour (Wire, or a set of

Edges) + mode of closure (by vertex or by edge)

\image html repair3.png

Our <b>TUI Scripts</b> provide you with useful examples of the use of

\ref tui_close_contour "Repairing Operations".

*/