/*!

\page create_squareface_page Square Face

A primitive called <b>Square Face</b> is a square plane of certain dimensions and orientation

To create a \b Square Face  in the <b>Main Menu</b> select <b>New Entity - >

Primitives - > Face</b>

\n There are 2 algorithms to create a \b Square \b Face in the 3D space.

\n The \b Result of each operation will be a GEOM_Object (Face).

\n Firstly, you can create a \b Face at the origin

of coordinates defining its boundaries by the height and the width and its axis by the orientation

radio buttons (OXY, OYZ or OZX).This

means that the Square Face will lie in "OXY", "OYZ" or "OZX" plane correspondingly.

\n <b>TUI Command:</b> <em>geompy.MakeFaceHW(Height, Width, Orientation)</em>

\n <b>Arguments:</b> Name + 3 values (Dimensions at origin: heigth, width and

orientation).

\image html face1.png

\n Secondly, you can define a \b Face by the height and the width sizes and an \b Edge defining the normal to the center of the face.

\n <b>TUI Command:</b> <em>geompy.MakeFaceObjHW(Vector, Height, Width)</em>

\n <b>Arguments:</b> Name + 1 Vector (normal to the center) + 2 doubles (to

describe a face sizes).

\n It is also possible to create a Square Face from another selected face. For this, switch the <b>Object Type</b> button

to \b Face, then select a face object and set the values of

Height and Width for the new face. The created face will lie in the plane of the selected face.

\n <b>TUI Command:</b> <em>geompy.MakeFaceObjHW(Face, Height, Width)</em>

\n <b>Arguments:</b> Name + 1 Face + 2 parameters (to describe a face sizes).

\image html face2.png

<b>Example:</b>

\image html faces.png "Square face"

Our <b>TUI Scripts</b> provide you with useful examples of creating

\ref tui_creation_face "Primitives".

*/