/*!

\page sewing_meshes_page Sewing meshes

\n In SMESH you can sew elements of  different meshes. The current

functionality allows you to sew:

<ul>

<li>\ref free_borders_anchor "Free borders"</li>

<li>\ref conform_free_borders_anchor "Conform free borders"</li>

<li>\ref border_to_side_anchor "Border to side"</li>

<li>\ref side_elements_anchor "Side elements"</li>

</ul>

\image html sewing.png "Sewing button"

<em>To sew elements of different meshes:</em>

<ol>

<li>From the \b Modification menu choose the \b Transformation item

and from its sub-menu select the \b Sewing item.</li>

<li>Check in the dialog box one of the radio buttons corresponding to

the type of sewing operation you would like to perform.</li>

<li>Fill the other fields available in the dialog box.</li>

<li>Click the \b Apply or <b>Apply and Close</b> button to perform the operation of sewing.</li>

</ol>

<br>

\anchor free_borders_anchor

<h2>Sew free borders</h2>

This functionality allows you to unite two free borders of a 2D mesh.

\image html sewing1.png

For sewing free borders you should define three points on each border:

first, second and the last node:

<ul>

<li>the first node specifies beginning of the border;</li>

<li>the second node specifies the part of the border which should be

considered (as far as the free border usually forms a closed

contour);</li>

<li>the last node specifies the end of the border.</li>

</ul>

You can select these nodes in the 3D viewer or define by its id.

The first and the second nodes should belong to the same link of a

face. The second and the last nodes of a border can be the same. The

first and the last nodes of two borders can be the same. The

corresponding end nodes of two borders will be merged. Intermediate

nodes of two borders will be either merged or inserted into faces of

the opposite border.

The sewing algorithm is as follows:

<ol>

<li>The parameter (U) of each node within a border is computed. So

that the first node has U=0.0, the last node has U=1.0, for the rest

nodes 0.0 < U < 1.0;</li>

<li>Compare node parameters of the two borders. If two nodes of the

opposite borders have close parameters, they are merged, i.e. a node

of the first border is replaced in all elements by a node of the

second border. If a node has no node with a close parameter in the

opposite border, it is inserted into an edge of element of the

opposite border, an element is split. Two nodes are considered close

enough to merge, if difference of their parameters is less than one

fifth of minimum length of adjacent face edges on the borders.</li>

</ol>

\image html image22.jpg "Sewing free borders"

<b>See Also</b> a sample TUI Script of a

\ref tui_sew_free_borders "Sew Free Borders" operation.

<br>

\anchor conform_free_borders_anchor

<h2>Sew conform free borders</h2>

This functionality can be used to unite two free borders of a 2D mesh.

\image html sewing2.png

The borders of meshes for sewing are defined as for "Sew free borders"

except that the second free border is not limited and can be defined

by the first and the second nodes only. The first nodes of two borders

can be the same.

The algorithm is following: counting nodes starting at the first ones,

the n-th node of the first border is merged with the n-th node of the

other border, until the end of either of borders. Nodes of the first

border are replaced in all elements with corresponding nodes of the

second border.

\n For sewing conform free borders you should define three points on

the first border and two points on the second one. User can select

these nodes in 3D viewer or define node by its id.

\image html image23.jpg "Sewing conform free borders"

<b>See Also</b> a sample TUI Script of a

\ref tui_sew_conform_free_borders "Sew Conform Free Borders"

operation.

<br>

\anchor border_to_side_anchor

<h2>Sew border to side</h2>

"Sew border to side" is intended to sew a free border to a mesh

surface.

\n The free border is defined as for "Sewing of free borders". The

place where to sew the border is defined by two nodes, between which

the border faces are placed, so that the first border node is merged

with the first node on the side and the last node of the border is

merged with the second specified node on the side.

\image html sewing3.png

The algorithm is following.

<ol>

<li>Find a sequence of linked nodes on the side such that the found

links to be most co-directed with the links of the free border.</li>

<li>Sew two sequences of nodes using algorithm of "Sewing of free berders".

\n For sewing border to side you should define three points on the

border and two points on the side. User can select these nodes in 3D

viewer or define node by its id.</li>

</ol>

\image html image30.jpg "Sewing border to side"

<b>See Also</b> a sample TUI Script of a

\ref tui_sew_meshes_border_to_side "Sew Border to Side" operation.

<br>

\anchor side_elements_anchor

<h2>Sew side elements</h2>

This operation is intended to unite two mesh surfaces.

\image html sewing4.png

Surfaces may be defined by either 2d or 3d elements. The number of

given elements of the sides must be the same. The sets of given

elements must be topologically equal, i.e. each node of one element

set must have a corresponding node in the other element set and

corresponding nodes must be equally linked. If there are 3d elements

in a set, only their free faces must obey to that rule.

\n Two corresponding nodes on each side must be specified. They must

belong to one element and must be located on an element set boundary.

Sewing algorithm finds and merges the corresponding nodes starting

from the specified ones.

\image html image31.jpg "Step-by-step sewing process"

\image html image32.jpg "The result of side elements sewing"

For sewing side elements you should define elements for sewing and two

nodes for merging on the each side. User can select these elements and

nodes in 3D viewer or define them by its id.

<b>See Also</b> a sample TUI Script of a

\ref tui_sew_side_elements "Sew Side Elements" operation.

*/