/*!

\page smoothing_page Smoothing

\n Smoothing is used to adjust the locations of element corners

(nodes) to reduce distortions in these elements.

<em>To apply smoothing to the elements of your mesh:</em>

<ol>

<li>In the \b Modification menu select the \b Smoothing item or click

<em>"Smoothing"</em> button in the toolbar.

\image html image84.png

<center><em>"Smoothing" button</em></center>

The following dialog will appear:

\image html smoothing.png

</li>

<li>In this dialog:

<ul>

<li>specify the IDs of the elements which will be smoothed:

<ul>

<li><b>Select the whole mesh, submesh or group</b> activating this

checkbox; or</li>

<li>choose mesh elements with the mouse in the 3D Viewer. It is

possible to select a whole area with a mouse frame; or</li>

<li>input the element IDs directly in <b>ID Elements</b> field. The selected elements will be highlighted in the

viewer; or</li>

<li>apply Filters. <b>Set filter</b> button allows to apply a filter to the selection of elements. See more

about filters in the \ref selection_filter_library_page "Selection filter library" page.</li>

</ul>

</li>

<li>define the <b>Fixed nodes ids</b> that should remain at their location during

smoothing. If a mesh is built on a shape, the nodes built on its

geometric edges are always fixed. If the smoothing is applied to a part

of the mesh, the nodes of boundary elements are also

fixed. It is possible to additionally fix any other nodes. The fixed nodes can be

selected manually or by filters, just as the smoothed elements.</li>

<li>choose the <b>Smoothing Method:</b>

<ul>

<li>\b Laplacian smoothing pulls a node toward the center of

surrounding nodes directly connected to that node along an element

edge.

<li>\b Centroidal smoothing pulls a node toward the

element-area-weighted centroid of the surrounding elements. </li>

Typically, the  Laplacian method will produce the mesh with the least element

distortion. It is also the fastest method. Centroidal smoothing usually

produces  a mesh with more uniform element sizes. Both methods

produce good results with "free" meshes.</li>

</ul>

\image html image83.gif

</li>

<li>specify the <b>Iteration limit</b>. Both smoothing methods

iterate through a number of steps to produce the resulting smoothed

mesh. At each new step the smoothing is reevaluated with the updated nodal locations. This

process continues till the limit of iterations has been

exceeded, or till the aspect ratio of all element is less than or equal to the

specified one.</li>

<li>specify the  <b>Max. aspect ratio</b> - the target mesh quality at which the

smoothing algorithm should stop the iterations.</li>

<li>activate <b>in parametric space</b> checkbox if it is necessary to

to improve the shape of faces in the parametric space

of geometrical surfaces on which they are generated, else the shape of

faces in the 3D space is improved. </li>

</ul>

</li>

<li>Click \b Apply or <b> Apply and Close</b> button to confirm the operation.</li>

</ol>

\image html smoothing1.png "The initial mesh"

\image html smoothing2.png "The smoothed mesh"

<br><b>See Also</b> a sample TUI Script of a

\ref tui_smoothing "Smoothing" operation.

*/