/*!

\page tui_defining_hypotheses_page Defining Hypotheses and Algorithms

<h2>Defining 1D Hypotheses</h2>

<br>

\anchor tui_1d_arithmetic

<h3>1D Arithmetic</h3>

\code

import geompy

import smesh

# create a box

box = geompy.MakeBoxDXDYDZ(10., 10., 10.)

geompy.addToStudy(box, "Box")

# create a hexahedral mesh on the box

hexa = smesh.Mesh(box, "Box : hexahedrical mesh")

# create a Regular 1D algorithm for edges

algo1D = hexa.Segment()

# optionally reverse node distribution on certain edges

allEdges = geompy.SubShapeAllSortedIDs( box, geompy.ShapeType["EDGE"])

reversedEdges = [ allEdges[0], allEdges[4] ]

# define "Arithmetic1D" hypothesis to cut all edges in several segments with increasing arithmetic length

algo1D.Arithmetic1D(1, 4, reversedEdges)

# create a quadrangle 2D algorithm for faces

hexa.Quadrangle()

# create a hexahedron 3D algorithm for solids

hexa.Hexahedron()

# compute the mesh

hexa.Compute()

\endcode

<br>

\anchor tui_deflection_1d

<h3>Deflection 1D and Number of Segments</h3>

\code

import geompy

import smesh

# create a face from arc and straight segment

px = geompy.MakeVertex(100., 0.  , 0.  )

py = geompy.MakeVertex(0.  , 100., 0.  )

pz = geompy.MakeVertex(0.  , 0.  , 100.)

exy = geompy.MakeEdge(px, py)

arc = geompy.MakeArc(py, pz, px)

wire = geompy.MakeWire([exy, arc])

isPlanarFace = 1

face1 = geompy.MakeFace(wire, isPlanarFace)

geompy.addToStudy(face1,"Face1")

# get edges from the face

e_straight,e_arc = geompy.SubShapeAll(face1, geompy.ShapeType["EDGE"])

geompy.addToStudyInFather(face1, e_arc, "Arc Edge")

# create hexahedral mesh

hexa = smesh.Mesh(face1, "Face : triangle mesh")

# define "NumberOfSegments" hypothesis to cut a straight edge in a fixed number of segments

algo1D = hexa.Segment()

algo1D.NumberOfSegments(6)

# define "MaxElementArea" hypothesis

algo2D = hexa.Triangle()

algo2D.MaxElementArea(70.0)

# define a local "Deflection1D" hypothesis on the arc

algo_local = hexa.Segment(e_arc)

algo_local.Deflection1D(1.0)

# compute the mesh

hexa.Compute()

\endcode

<br>

\anchor tui_start_and_end_length

<h3>Start and End Length</h3>

\code

from geompy import *

import smesh

# create a box

box = MakeBoxDXDYDZ(10., 10., 10.)

addToStudy(box, "Box")

# get one edge of the box to put local hypothesis on

p5 = MakeVertex(5., 0., 0.)

EdgeX = GetEdgeNearPoint(box, p5)

addToStudyInFather(box, EdgeX, "Edge [0,0,0 - 10,0,0]")

# create a hexahedral mesh on the box

hexa = smesh.Mesh(box, "Box : hexahedrical mesh")

# set algorithms

algo1D = hexa.Segment()

hexa.Quadrangle()

hexa.Hexahedron()

# define "NumberOfSegments" hypothesis to cut an edge in a fixed number of segments

algo1D.NumberOfSegments(4)

# create a local hypothesis

algo_local = hexa.Segment(EdgeX)

# define "StartEndLength" hypothesis to cut an edge in several segments with increasing geometric length

algo_local.StartEndLength(1, 6)

# define "Propagation" hypothesis that propagates all other hypothesis

# on all edges on the opposite side in case of quadrangular faces

algo_local.Propagation()

# compute the mesh

hexa.Compute()

\endcode

<br>

\anchor tui_average_length

<h3>Average Length</h3>

\code

from geompy import *

import smesh

# create a box

box = MakeBoxDXDYDZ(10., 10., 10.)

addToStudy(box, "Box")

# get one edge of the box to put local hypothesis on

p5 = MakeVertex(5., 0., 0.)

EdgeX = GetEdgeNearPoint(box, p5)

addToStudyInFather(box, EdgeX, "Edge [0,0,0 - 10,0,0]")

# create a hexahedral mesh on the box

hexa = smesh.Mesh(box, "Box : hexahedrical mesh")

# set algorithms

algo1D = hexa.Segment()

hexa.Quadrangle()

hexa.Hexahedron()

# define "NumberOfSegments" hypothesis to cut all edges in a fixed number of segments

algo1D.NumberOfSegments(4)

# create a sub-mesh

algo_local = hexa.Segment(EdgeX)

# define "LocalLength" hypothesis to cut an edge in several segments with the same length

algo_local.LocalLength(2.)

# define "Propagation" hypothesis that propagates all other hypothesis

# on all edges on the opposite side in case of quadrangular faces

algo_local.Propagation()

# compute the mesh

hexa.Compute()

\endcode

<br><h2>Defining 2D and 3D hypotheses</h2>

<br>

\anchor tui_max_element_area

<h3>Maximum Element Area</h3>

\code

import geompy

import smesh

import salome

# create a face

px   = geompy.MakeVertex(100., 0.  , 0.  )

py   = geompy.MakeVertex(0.  , 100., 0.  )

pz   = geompy.MakeVertex(0.  , 0.  , 100.)

vxy = geompy.MakeVector(px, py)

arc = geompy.MakeArc(py, pz, px)

wire = geompy.MakeWire([vxy, arc])

isPlanarFace = 1

face = geompy.MakeFace(wire, isPlanarFace)

# add the face in the study

id_face = geompy.addToStudy(face, "Face to be meshed")

# create a mesh

tria_mesh = smesh.Mesh(face, "Face : triangulation")

# define 1D meshing:

algo = tria_mesh.Segment()

algo.NumberOfSegments(20)

# define 2D meshing:

# assign triangulation algorithm

algo = tria_mesh.Triangle()

# apply "Max Element Area" hypothesis to each triangle

algo.MaxElementArea(100)

# compute the mesh

tria_mesh.Compute()

\endcode

<br>

\anchor tui_max_element_volume

<h3>Maximum Element Volume</h3>

\code

import geompy

import smesh

# create a cylinder

cyl = geompy.MakeCylinderRH(30., 50.)

geompy.addToStudy(cyl, "cyl")

# create a mesh on the cylinder

tetra = smesh.Mesh(cyl, "Cylinder : tetrahedrical mesh")

# assign algorithms

algo1D = tetra.Segment()

algo2D = tetra.Triangle()

algo3D = tetra.Tetrahedron(smesh.NETGEN)

# assign 1D and 2D hypotheses

algo1D.NumberOfSegments(7)

algo2D.MaxElementArea(150.)

# assign Max Element Volume hypothesis

algo3D.MaxElementVolume(200.)

# compute the mesh

ret = tetra.Compute()

if ret == 0:

print "probleme when computing the mesh"

else:

print "Computation succeded"

\endcode

<br>

\anchor tui_length_from_edges

<h3>Length from Edges</h3>

\code

import geompy

import smesh

# create sketchers

sketcher1 = geompy.MakeSketcher("Sketcher:F 0 0:TT 70 0:TT 70 70:TT 0 70:WW")

sketcher2 = geompy.MakeSketcher("Sketcher:F 20 20:TT 50 20:TT 50 50:TT 20 50:WW")

# create a face from two wires

isPlanarFace = 1

face1 = geompy.MakeFaces([sketcher1, sketcher2], isPlanarFace)

geompy.addToStudy(face1, "Face1")

# create a mesh

tria = smesh.Mesh(face1, "Face : triangle 2D mesh")

# Define 1D meshing

algo1D = tria.Segment()

algo1D.NumberOfSegments(2)

# create and assign the algorithm for 2D meshing with triangles

algo2D = tria.Triangle()

# create and assign "LengthFromEdges" hypothesis to build triangles based on the length of the edges taken from the wire

algo2D.LengthFromEdges()

# compute the mesh

tria.Compute()

\endcode

<br><h2>Defining Additional Hypotheses</h2>

<br>

\anchor tui_propagation

<h3>Propagation</h3>

\code

from geompy import *

import smesh

# create a box

box = MakeBoxDXDYDZ(10., 10., 10.)

addToStudy(box, "Box")

# get one edge of the box to put local hypothesis on

p5 = MakeVertex(5., 0., 0.)

EdgeX = GetEdgeNearPoint(box, p5)

addToStudyInFather(box, EdgeX, "Edge [0,0,0 - 10,0,0]")

# create a hexahedral mesh on the box

hexa = smesh.Mesh(box, "Box : hexahedrical mesh")

# set global algorithms and hypotheses

algo1D = hexa.Segment()

hexa.Quadrangle()

hexa.Hexahedron()

algo1D.NumberOfSegments(4)

# create a sub-mesh with local 1D hypothesis and propagation

algo_local = hexa.Segment(EdgeX)

# define "Arithmetic1D" hypothesis to cut an edge in several segments with increasing length

algo_local.Arithmetic1D(1, 4)

# define "Propagation" hypothesis that propagates all other 1D hypotheses

# from all edges on the opposite side of a face in case of quadrangular faces

algo_local.Propagation()

# compute the mesh

hexa.Compute()

\endcode

<br>

\anchor tui_defining_meshing_algos

<h2>Defining Meshing Algorithms</h2>

\code

import geompy

import smesh

# create a box

box = geompy.MakeBoxDXDYDZ(10., 10., 10.)

geompy.addToStudy(box, "Box")

# 1. Create a hexahedral mesh on the box

hexa = smesh.Mesh(box, "Box : hexahedrical mesh")

# create a Regular 1D algorithm for edges

algo1D = hexa.Segment()

# create a quadrangle 2D algorithm for faces

algo2D = hexa.Quadrangle()

# create a hexahedron 3D algorithm for solids

algo3D = hexa.Hexahedron()

# define hypotheses

algo1D.Arithmetic1D(1, 4)

# compute the mesh

hexa.Compute()

# 2. Create a tetrahedral mesh on the box

tetra = smesh.Mesh(box, "Box : tetrahedrical mesh")

# create a Regular 1D algorithm for edges

algo1D = tetra.Segment()

# create a Mefisto 2D algorithm for faces

algo2D = tetra.Triangle()

# create a Netgen 3D algorithm for solids

algo3D = tetra.Tetrahedron(smesh.NETGEN)

# define hypotheses

algo1D.Arithmetic1D(1, 4)

algo2D.LengthFromEdges()

# compute the mesh

tetra.Compute()

# 3. Create a tetrahedral mesh on the box with NETGEN_2D3D algorithm

tetraN = smesh.Mesh(box, "Box : tetrahedrical mesh by NETGEN_2D3D")

# create a Netgen_2D3D algorithm for solids

algo3D = tetraN.Tetrahedron(smesh.FULL_NETGEN)

# define hypotheses

n23_params = algo3D.Parameters()

# compute the mesh

tetraN.Compute()

\endcode

<br>

\anchor tui_projection

<h3>Projection Algorithms</h3>

\code

# Project prisms from one meshed box to another mesh on the same box

from smesh import *

# Prepare geometry

# Create a parallelepiped

box = geompy.MakeBoxDXDYDZ(200, 100, 70)

geompy.addToStudy( box, "box" )

# Get geom faces to mesh with triangles in the 1ts and 2nd meshes

faces = geompy.SubShapeAll(box, geompy.ShapeType["FACE"])

# 2 adjacent faces of the box

f1 = faces[2]

f2 = faces[0]

# face opposite to f2

f2opp = faces[1]

# Get vertices used to specify how to associate sides of faces at projection

[v1F1, v2F1] = geompy.SubShapeAll(f1, geompy.ShapeType["VERTEX"])[:2]

[v1F2, v2F2] = geompy.SubShapeAll(f2, geompy.ShapeType["VERTEX"])[:2]

geompy.addToStudyInFather( box, v1F1, "v1F1" )

geompy.addToStudyInFather( box, v2F1, "v2F1" )

geompy.addToStudyInFather( box, v1F2, "v1F2" )

geompy.addToStudyInFather( box, v2F2, "v2F2" )

# Make group of 3 edges of f1 and f2

edgesF1 = geompy.CreateGroup(f1, geompy.ShapeType["EDGE"])

geompy.UnionList( edgesF1, geompy.SubShapeAll(f1, geompy.ShapeType["EDGE"])[:3])

edgesF2 = geompy.CreateGroup(f2, geompy.ShapeType["EDGE"])

geompy.UnionList( edgesF2, geompy.SubShapeAll(f2, geompy.ShapeType["EDGE"])[:3])

geompy.addToStudyInFather( box, edgesF1, "edgesF1" )

geompy.addToStudyInFather( box, edgesF2, "edgesF2" )

# Make the source mesh with prisms

src_mesh = Mesh(box, "Source mesh")

src_mesh.Segment().NumberOfSegments(9,10)

src_mesh.Quadrangle()

src_mesh.Hexahedron()

src_mesh.Triangle(f1) # triangular sumbesh

src_mesh.Compute()

# Mesh the box using projection algoritms

# Define the same global 1D and 2D hypotheses

tgt_mesh = Mesh(box, "Target mesh")

tgt_mesh.Segment().NumberOfSegments(9,10,UseExisting=True)

tgt_mesh.Quadrangle()

# Define Projection 1D algorithm to project 1d mesh elements from group edgesF2 to edgesF1

# It is actually not needed, just a demonstration

proj1D = tgt_mesh.Projection1D( edgesF1 )

# each vertex must be at the end of a connected group of edges (or a sole edge)

proj1D.SourceEdge( edgesF2, src_mesh, v2F1, v2F2 )

# Define 2D hypotheses to project triangles from f1 face of the source mesh to

# f2 face in the target mesh. Vertices specify how to associate sides of faces

proj2D = tgt_mesh.Projection2D( f2 )

proj2D.SourceFace( f1, src_mesh, v1F1, v1F2, v2F1, v2F2 )

# 2D hypotheses to project triangles from f2 of target mesh to the face opposite to f2.

# Association of face sides is default

proj2D = tgt_mesh.Projection2D( f2opp )

proj2D.SourceFace( f2 )

# 3D hypotheses to project prisms from the source to the target mesh

proj3D = tgt_mesh.Projection3D()

proj3D.SourceShape3D( box, src_mesh, v1F1, v1F2, v2F1, v2F2 )

tgt_mesh.Compute()

# Move the source mesh to visualy compare the two meshes

src_mesh.TranslateObject( src_mesh, MakeDirStruct( 210, 0, 0 ), Copy=False)

\endcode

<br>

\anchor tui_fixed_points

<h2>1D Mesh with Fixed Points example</h2>

\code

import salome

import geompy

import smesh

import StdMeshers

# Create face and explode it on edges

face = geompy.MakeFaceHW(100, 100, 1)

edges = geompy.SubShapeAllSorted(face, geompy.ShapeType["EDGE"])

geompy.addToStudy( face, "Face" )

# get the first edge from exploded result

edge1 = geompy.GetSubShapeID(face, edges[0])

# Define Mesh on previously created face

Mesh_1 = smesh.Mesh(face)

# Create Fixed Point 1D hypothesis and define parameters.

# Note: values greater than 1.0 and less than 0.0 are not taken into account;

# duplicated values are removed. Also, if not specified explicitly, values 0.0 and 1.0

# add added automatically.

# The number of segments should correspond to the number of points (NbSeg = NbPnt-1);

# extra values of segments splitting parameter are not taken into account,

# while missing values are considered to be equal to 1.

Fixed_points_1D_1 = smesh.CreateHypothesis('FixedPoints1D')

Fixed_points_1D_1.SetPoints( [ 1.1, 0.9, 0.5, 0.0, 0.5, -0.3 ] )

Fixed_points_1D_1.SetNbSegments( [ 3, 1, 2 ] )

Fixed_points_1D_1.SetReversedEdges( [edge1] )

# Add hypothesis to mesh and define 2D parameters

Mesh_1.AddHypothesis(Fixed_points_1D_1)

Regular_1D = Mesh_1.Segment()

Quadrangle_2D = Mesh_1.Quadrangle()

# Compute mesh

Mesh_1.Compute()

\endcode

\anchor tui_radial_quadrangle

<h2> Radial Quadrangle 1D2D example </h2>

\code

import salome

import geompy

import smesh

import StdMeshers

# Create face from the wire and add to study

WirePath = geompy.MakeSketcher("Sketcher:F 0 0:TT 20 0:R 90:C 20 90:WW", [0, 0, 0, 1, 0, 0, 0, 0, 1])

Face = geompy.MakeFace(WirePath,1)

geompy.addToStudy(Face,"Face")

# Define geometry for mesh, and 1D parameters

mesh = smesh.Mesh(Face)

Wire_discretisation = mesh.Segment()

Nb_Segments = Wire_discretisation.NumberOfSegments(5)

Nb_Segments.SetDistrType( 0 )

# Define 2D parameters and Radial Quadrange hypothesis

Number_of_Layers = smesh.CreateHypothesis('NumberOfLayers2D')

Number_of_Layers.SetNumberOfLayers( 4 )

mesh.AddHypothesis(Number_of_Layers)

RadialQuadrangle_1D2D = smesh.CreateHypothesis('RadialQuadrangle_1D2D')

mesh.AddHypothesis(RadialQuadrangle_1D2D)

mesh.Compute()

\endcode

\n Other meshing algorithms:

<ul>

<li>\subpage tui_defining_blsurf_hypotheses_page</li>

<li>\subpage tui_defining_ghs3d_hypotheses_page</li>

</ul>

*/