/*!

\page types_of_gauss_points_presentations_page Types of Gauss Points Presentations

\n There exist three different types of Gauss Points Presentations:

<b>Results at Gauss Points</b>, <b>Gauss Points on Geometry</b> and

<b>Gauss Points on Deformed Shape</b>. They all can be accessed from

the same menu.

<br><h2>Results at Gauss Points</h2>

\image html image31.jpg

To create a <b>Results at Gauss Points</b> presentation, select \b

Results radio button. This presentation gives most information as the

points here vary in color and size depending on the value.

\n As for the size, the minimum and maximum <b>Range values</b> for

the rendering can be defined in the configuration dialog. These values

are multiplied by a reference length (representative of the average

size of cells of the mesh) to define the range for minimum and maximum

size of a point during rendering (at magnification = 100%). Default

values are:

<ul>

<li>For the \b Rainbow scale: min = 10%, max = 33%</li>

<ul>

<li>Min. size will be associated to the smallest real value (included

negative values) among the results.</li>

<li>Max size will be associated to the largest real values among the

results.</li>

</ul>

<li>For the bicolor scale: min = 0 (not editable), max = 33%</li>

<ul>

<li>Null size will be associated to the 0 scalar value of a

result.</li>

<li>Max size will be associated to the largest absolute values of

results.</li>

</ul>

</ul>

Both values are dynamically updated by the system according to the

selected scalar bar. In the case of a \b Bicolor scale, the minimum

value is set to 0 in the dialog and the control is disabled.

\n <b>Magnification (%)</b> corresponds to the change of size of results

at Gauss point primitives in 2D space. Acceptable values range from 0

to N; 100% means no magnification, 50% means half of its size, 200%

mean twice its size and so forth. By default this value is set to

100%.

\n <b>+/- Ratio</b> corresponds to the number by which the magnification

will be respectively multiplied or divided at edition, ranging from

0.01 to 10. By default this value is set to 2.

\n During rendering of the scene, it is possible to modify the

magnification (multiply/divide by the magnification ratio) by using

?M? and ?m? keys of the keyboard when the 3D view is active. The

effect is applied immediately. The ratio is only defined through the

dialog box.

<br><h2>Gauss Points on Geometry</h2>

\image html gpp2.png

To create a \b Geometry presentation, select the respective radio

button. This presentation shows only geometrical position of Gauss

Points as their size and color are fixed (because the results are not

computed).

\n \b Size defines a value that will be multiplied by a reference

length (representative of the average size of cells of the mesh) to

define the size of points during rendering (at magnification =

100%). Default values 10%.

\n \b Magnification and \b Ratio have the same meaning as for Results

at Gauss points presentation.

\n \b Color allows to define the unique color for Gauss Points in a

standard color configuration dialog.

<br><h2>Gauss Points on Deformed Shape</h2>

\image html gpp3.png

To create a <b>Deformed Shape</b> presentation, select the respective

radio button. This presentation is analog to <b>Results at Gauss

Points</b> except that the shape on which the points are located is

deformed using the <b>Scale Factor</b> and the Gauss Points are

processed only after that.

*/